Datacenter Network Simulator

Release Notes — Version 2.1

Release Date: May 11, 2026

A visual datacenter topology simulator generating authentic SNMP, gNMI,
and sFlow telemetry for realistic infrastructure testing.

# 1. Overview

Version 2.1 is a major feature release for the Datacenter Network Simulator. It introduces an event-driven rule engine that drives SNMP trap generation across all device types, a fully separated management and power infrastructure model, physical rack location awareness, and a three-layer topology visualization. Together these features bring the simulator substantially closer to a real datacenter's operational behavior, making it suitable for testing NMS integrations, alert workflows, and SNMP toolchain pipelines.

Key themes of this release:

Rule-driven SNMP trap generation — 30+ configurable rules, 16 trap types

Separated production, management (OOB), and power network planes

New device classes: UPS, Rack PDU, Floor PDU, OOB Switch, Environmental Sensor

Physical rack-location awareness and cross-device rack failure correlation

Three-layer interactive topology canvas with per-layer filtering

# 2. Rule-Based SNMP Trap Engine

The trap engine is the most significant new subsystem in v2.1. A lightweight Rete-inspired rule evaluator runs on every simulation tick, consuming a structured DeviceFact snapshot for each device and evaluating every enabled rule against it. When a rule matches, an SNMPv2c trap is dispatched asynchronously to a configurable receiver. The engine operates on a background thread so it never blocks the Qt UI.

All rule logic, trap OIDs, severities, and device-type filters are fully configurable via trap_rules.json or the in-application Rules Panel — no code changes required.

## 2.1  Device Fact Model

Each evaluation cycle, the device state store emits one DeviceFact per device. The fact is a point-in-time, typed snapshot of all metrics that rules may inspect:

| Field | Type | Description |
| --- | --- | --- |
| device_id | str | Unique device name within the topology |
| device_type | str | router ¦ switch ¦ server ¦ firewall ¦ load_balancer ¦ ups ¦ pdu ¦ floor_pdu ¦ oob_switch ¦ sensor |
| ip_address | str | Primary (production) IPv4 address |
| timestamp | float | Unix epoch seconds when the fact was generated |
| cpu_usage | float | CPU utilisation 0–100 % |
| memory_usage | float | Memory utilisation 0–100 % |
| disk_usage | float | Disk utilisation 0–100 % |
| temperature | float | Chassis / ASIC temperature (°C) |
| ambient_temp | float | Datacenter inlet/ambient temperature (°C) — sensors only |
| humidity | float | Relative humidity % — sensors only |
| dewpoint | float | Dew-point temperature (°C) — Vertiv Geist sensors |
| airflow | float | Airflow velocity (m/s) — APC NetBotz sensors |
| ups_status | str | normal ¦ on_battery ¦ low_battery — UPS devices |
| interfaces | list | Per-interface snapshot: index, name, oper_status, bandwidth_util, error_rate |
| bgp_sessions | list | Per-peer snapshot: peer_addr, state — routers and firewalls |
| rack_id | str | Physical rack identifier, e.g. DC1:R1:RACK3 — used by rack failure rule |
| datacenter | str | Datacenter name for cross-device correlation |

## 2.2  Rule Types

Five distinct condition types are supported. Each type is evaluated by a dedicated evaluator inside the rule engine, allowing precise control over when a trap fires.

Threshold

Fires when a scalar metric satisfies a comparison operator against a fixed value. An optional duration_sec parameter requires the condition to remain true continuously for the specified number of seconds before the trap fires. This prevents transient spikes from generating noise.

Operators: >  <  >=  <=  ==  !=

Example: cpu_usage > 90.0 for 300 seconds (5-minute sustained CPU alert)

Example: temperature > 60.0 (immediate chassis overtemperature alert)

State Change

Fires when a tracked metric transitions from one discrete state to another. Either the from_state or to_state may be set to null to match any origin or any destination respectively, enabling wildcard transitions.

Tracked metrics: interface_status, ups_status, bgp_session

Example: interface_status  up → down  (link-down trap)

Example: ups_status  null → low_battery  (any state to low battery)

Temporal

Fires when a named event occurs at least N times within a sliding time window. Timestamps are maintained in a per-device per-rule deque. Entries older than window_sec are expired before each evaluation, giving an accurate count of recent events.

Parameters: event_type, event_count (N), window_sec

Example: linkDown event  3 times in 60 seconds → link flap trap

Composite

Combines two or more sub-conditions with AND or OR logic. Sub-conditions may be any scalar (threshold) condition. Composite rules allow multi-metric correlation within a single device — for instance, firing only when CPU and temperature are simultaneously elevated.

Logic: AND | OR

Example: cpu_usage > 90% AND temperature > 60°C → critical combined alert

Example: humidity < 30% OR humidity > 70% → out-of-range humidity

Rack Failure

A cross-device correlation rule. It groups all devices sharing the same rack_id and fires a single aggregated trap when the number of impaired devices in that rack reaches or exceeds the configured threshold. This prevents flooding a receiver with per-device alerts when a PDU or power feed fails.

Parameter: threshold (minimum number of impaired devices; default 3)

rack_id format: datacenter:row:rack, e.g. DC1:R1:RACK3

Fires one RACK_FAILURE trap per rack, not per device

## 2.3  Recovery Rules

Any rule can be marked as a recovery rule by setting is_recovery=true and referencing the originating alert rule via recovery_of. A recovery rule fires when its condition is met after the referenced alert rule was active, providing clear event pairs to NMS platforms. Recovery traps are sent with informational severity using OID 1.3.6.1.6.3.1.1.5.4 (linkUp) as the generic clearance signal.

Shipped recovery rules:

| Recovery Rule | Condition | Clears Alert |
| --- | --- | --- |
| CPUNormal | cpu_usage < 70% | CPUHigh / HighCPUSustained |
| MemoryNormal | memory_usage < 70% | HighMemory |
| TemperatureNormal | temperature < 55°C | HighTemperature |
| SensorAmbientTempNormal | ambient_temp < 28°C | SensorAmbientTempHigh |
| SensorHumidityNormal | humidity >= 30% AND <= 70% | SensorHighHumidity |
| SensorDewPointNormal | dewpoint <= 17°C | SensorHighDewPoint |
| SensorAirflowNormal | airflow >= 0.3 m/s AND <= 3.5 m/s | SensorHighAirflow / LowAirflow |

# 3. SNMP Trap Catalog

Sixteen trap types are defined in v2.1. Each has a fixed OID, a default severity, a set of applicable device types, and a set of SNMPv2c varbinds that are automatically populated at dispatch time. The enterprise OID tree 1.3.6.1.4.1.99999 is used for resource and environmental traps not covered by standard MIBs.

## 3.1  Cold Start

| OID | 1.3.6.1.6.3.1.1.5.1 |
| --- | --- |
| Severity | INFORMATIONAL |
| Applicable Devices | All device types |

Device has powered on or recovered from a hard reset. Indicates the agent re-initialized all management data.

SNMPv2c Varbinds:

| OID | Type | Description |
| --- | --- | --- |
| 1.3.6.1.2.1.1.3.0 | TimeTicks | sysUpTime — centiseconds since agent start |
| 1.3.6.1.6.3.1.1.4.1.0 | OID | snmpTrapOID — 1.3.6.1.6.3.1.1.5.1 |
| 1.3.6.1.2.1.1.1.0 | OctetString | sysDescr — device hardware description |

## 3.2  Warm Start

| OID | 1.3.6.1.6.3.1.1.5.2 |
| --- | --- |
| Severity | INFORMATIONAL |
| Applicable Devices | Router, Switch, Firewall, Load Balancer, OOB Switch |

Device process or daemon restarted without a full power cycle. Management data may have been partially reset.

SNMPv2c Varbinds:

| OID | Type | Description |
| --- | --- | --- |
| 1.3.6.1.2.1.1.3.0 | TimeTicks | sysUpTime |
| 1.3.6.1.6.3.1.1.4.1.0 | OID | snmpTrapOID — 1.3.6.1.6.3.1.1.5.2 |
| 1.3.6.1.2.1.1.1.0 | OctetString | sysDescr |

## 3.3  Link Down

| OID | 1.3.6.1.6.3.1.1.5.3 |
| --- | --- |
| Severity | MAJOR |
| Applicable Devices | Router, Switch, Server, Firewall, Load Balancer, OOB Switch, PDU |

A network interface transitioned from operationally up to operationally down. Fired by the LinkDown state-change rule (interface_status: up → down).

SNMPv2c Varbinds:

| OID | Type | Description |
| --- | --- | --- |
| 1.3.6.1.2.1.1.3.0 | TimeTicks | sysUpTime |
| 1.3.6.1.6.3.1.1.4.1.0 | OID | snmpTrapOID — 1.3.6.1.6.3.1.1.5.3 |
| 1.3.6.1.2.1.2.2.1.1.{n} | Integer | ifIndex — interface index |
| 1.3.6.1.2.1.2.2.1.7.{n} | Integer | ifAdminStatus — 1=up, 2=down, 3=testing |
| 1.3.6.1.2.1.2.2.1.8.{n} | Integer | ifOperStatus — 2 (down) |
| 1.3.6.1.2.1.2.2.1.2.{n} | OctetString | ifDescr — interface name |

## 3.4  Link Up

| OID | 1.3.6.1.6.3.1.1.5.4 |
| --- | --- |
| Severity | INFORMATIONAL |
| Applicable Devices | Router, Switch, Server, Firewall, Load Balancer, OOB Switch, PDU |

A network interface transitioned from operationally down to operationally up. Also used as the generic clearance OID for recovery rules.

SNMPv2c Varbinds:

| OID | Type | Description |
| --- | --- | --- |
| 1.3.6.1.2.1.1.3.0 | TimeTicks | sysUpTime |
| 1.3.6.1.6.3.1.1.4.1.0 | OID | snmpTrapOID — 1.3.6.1.6.3.1.1.5.4 |
| 1.3.6.1.2.1.2.2.1.1.{n} | Integer | ifIndex |
| 1.3.6.1.2.1.2.2.1.7.{n} | Integer | ifAdminStatus — 1 (up) |
| 1.3.6.1.2.1.2.2.1.8.{n} | Integer | ifOperStatus — 1 (up) |
| 1.3.6.1.2.1.2.2.1.2.{n} | OctetString | ifDescr — interface name |

## 3.5  Authentication Failure

| OID | 1.3.6.1.6.3.1.1.5.5 |
| --- | --- |
| Severity | MAJOR |
| Applicable Devices | All device types |

An SNMP request was received carrying an incorrect community string. Useful for detecting misconfigured pollers or unauthorized access attempts.

SNMPv2c Varbinds:

| OID | Type | Description |
| --- | --- | --- |
| 1.3.6.1.2.1.1.3.0 | TimeTicks | sysUpTime |
| 1.3.6.1.6.3.1.1.4.1.0 | OID | snmpTrapOID — 1.3.6.1.6.3.1.1.5.5 |

## 3.6  BGP Session Down

| OID | 1.3.6.1.2.1.15.0.2 |
| --- | --- |
| Severity | CRITICAL |
| Applicable Devices | Router, Firewall, BGP-capable Switches (Nexus, QFX, Arista, Aruba 6300/8325, CE6/8) |

A BGP peer session transitioned from Established to Idle or Active. Fired by the BGPSessionDown state-change rule (bgp_session: established → idle). This is the standard bgpBackwardTransition trap from BGP4-MIB.

SNMPv2c Varbinds:

| OID | Type | Description |
| --- | --- | --- |
| 1.3.6.1.2.1.1.3.0 | TimeTicks | sysUpTime |
| 1.3.6.1.6.3.1.1.4.1.0 | OID | snmpTrapOID — 1.3.6.1.2.1.15.0.2 |
| 1.3.6.1.2.1.15.3.1.7.0 | IpAddress | bgpPeerRemoteAddr — peer IP address |
| 1.3.6.1.2.1.15.3.1.14.0 | Integer | bgpPeerState — 1=idle, 2=connect, 3=active, 4=openSent, 5=openConfirm, 6=established |

## 3.7  UPS On Battery

| OID | 1.3.6.1.2.1.33.2.0.1 |
| --- | --- |
| Severity | CRITICAL |
| Applicable Devices | UPS devices only |

The UPS switched from utility (mains) power to battery power, indicating a power feed interruption. Fired by the UPSOnBattery state-change rule (ups_status: normal → on_battery). Corresponds to the standard upsAlarmOnBattery notification from UPS-MIB (RFC 1628).

SNMPv2c Varbinds:

| OID | Type | Description |
| --- | --- | --- |
| 1.3.6.1.2.1.1.3.0 | TimeTicks | sysUpTime |
| 1.3.6.1.6.3.1.1.4.1.0 | OID | snmpTrapOID — 1.3.6.1.2.1.33.2.0.1 |
| 1.3.6.1.2.1.33.1.2.1.0 | Integer | upsBatteryStatus — 2=batteryNormal, 3=batteryLow, 4=batteryDepleted |

## 3.8  UPS Low Battery

| OID | 1.3.6.1.2.1.33.2.0.2 |
| --- | --- |
| Severity | CRITICAL |
| Applicable Devices | UPS devices only |

The UPS battery level has fallen to a critically low level. Remaining runtime may be insufficient to support an orderly shutdown. Fired by UPSLowBattery state-change rule (ups_status: any → low_battery). Corresponds to upsAlarmLowBattery from UPS-MIB.

SNMPv2c Varbinds:

| OID | Type | Description |
| --- | --- | --- |
| 1.3.6.1.2.1.1.3.0 | TimeTicks | sysUpTime |
| 1.3.6.1.6.3.1.1.4.1.0 | OID | snmpTrapOID — 1.3.6.1.2.1.33.2.0.2 |
| 1.3.6.1.2.1.33.1.2.1.0 | Integer | upsBatteryStatus — 3 (batteryLow) |

## 3.9  CPU High Usage

| OID | 1.3.6.1.4.1.99999.1.1 |
| --- | --- |
| Severity | MAJOR / CRITICAL |
| Applicable Devices | Router, Switch, Server, Firewall, Load Balancer, OOB Switch |

CPU utilisation exceeded the threshold (90%). Two rules target this trap: HighCPU fires immediately on first breach (major); HighCPUSustained fires only after the condition is held for 300 continuous seconds (critical). A composite variant CriticalCPUAndTemp fires when both CPU > 90% and temperature > 60°C are simultaneously true.

SNMPv2c Varbinds:

| OID | Type | Description |
| --- | --- | --- |
| 1.3.6.1.2.1.1.3.0 | TimeTicks | sysUpTime |
| 1.3.6.1.6.3.1.1.4.1.0 | OID | snmpTrapOID — 1.3.6.1.4.1.99999.1.1 |
| 1.3.6.1.4.1.99999.2.1 | Gauge32 | cimCpuUsage — current CPU % (×1, integer) |
| 1.3.6.1.4.1.99999.2.5 | Gauge32 | cimCpuThreshold — threshold value (90) |

## 3.10  Memory High Usage

| OID | 1.3.6.1.4.1.99999.1.2 |
| --- | --- |
| Severity | MAJOR |
| Applicable Devices | Router, Switch, Server, Firewall, Load Balancer |

Memory utilisation exceeded 85%. Fired by the HighMemory threshold rule. Servers are most likely to trigger this trap given their workload profiles. A recovery trap (MemoryNormal) is dispatched when utilisation drops below 70%.

SNMPv2c Varbinds:

| OID | Type | Description |
| --- | --- | --- |
| 1.3.6.1.2.1.1.3.0 | TimeTicks | sysUpTime |
| 1.3.6.1.6.3.1.1.4.1.0 | OID | snmpTrapOID — 1.3.6.1.4.1.99999.1.2 |
| 1.3.6.1.4.1.99999.2.2 | Gauge32 | cimMemUsage — current memory % (×1, integer) |
| 1.3.6.1.4.1.99999.2.6 | Gauge32 | cimMemThreshold — threshold value (85) |

## 3.11  Temperature Alert

| OID | 1.3.6.1.4.1.99999.1.3 |
| --- | --- |
| Severity | CRITICAL |
| Applicable Devices | Router, Switch, Server, Firewall, Load Balancer, UPS, OOB Switch, Sensor (ambient) |

Chassis or ASIC temperature exceeded 60°C (network gear), or ambient inlet temperature exceeded 32°C (sensors, major) / 38°C (sensors, critical). The composite CriticalCPUAndTemp rule can also fire this OID when CPU and temperature thresholds are simultaneously exceeded.

SNMPv2c Varbinds:

| OID | Type | Description |
| --- | --- | --- |
| 1.3.6.1.2.1.1.3.0 | TimeTicks | sysUpTime |
| 1.3.6.1.6.3.1.1.4.1.0 | OID | snmpTrapOID — 1.3.6.1.4.1.99999.1.3 |
| 1.3.6.1.4.1.99999.2.3 | Gauge32 | cimTempValue — current temperature °C (×1) |
| 1.3.6.1.4.1.99999.2.7 | Gauge32 | cimTempThreshold — threshold °C (60 or 32/38 for sensors) |

## 3.12  Link Flap

| OID | 1.3.6.1.4.1.99999.1.4 |
| --- | --- |
| Severity | CRITICAL |
| Applicable Devices | Router, Switch, Firewall |

A single interface cycled through link-down events three or more times within a 60-second sliding window. This indicates an unstable physical connection, bad optic, or misconfigured port. The temporal LinkFlap rule drives this trap. Each new linkDown event is pushed into the window; when the count reaches 3, the trap fires. The window resets after the trap is sent.

SNMPv2c Varbinds:

| OID | Type | Description |
| --- | --- | --- |
| 1.3.6.1.2.1.1.3.0 | TimeTicks | sysUpTime |
| 1.3.6.1.6.3.1.1.4.1.0 | OID | snmpTrapOID — 1.3.6.1.4.1.99999.1.4 |
| 1.3.6.1.4.1.99999.2.4 | Gauge32 | cimLinkFlapCount — number of flaps observed |
| 1.3.6.1.4.1.99999.2.8 | Gauge32 | cimLinkFlapWindow — observation window in seconds (60) |
| 1.3.6.1.2.1.2.2.1.1.{n} | Integer | ifIndex — index of the flapping interface |

## 3.13  Rack Failure

| OID | 1.3.6.1.4.1.99999.1.5 |
| --- | --- |
| Severity | CRITICAL |
| Applicable Devices | Correlated across all device types sharing a rack_id |

Three or more devices assigned to the same physical rack are simultaneously impaired (unreachable or in a degraded state). One aggregated trap is dispatched per affected rack rather than per device, reducing alert storms. This rule is particularly effective at detecting PDU or power feed failures that take down an entire rack.

SNMPv2c Varbinds:

| OID | Type | Description |
| --- | --- | --- |
| 1.3.6.1.2.1.1.3.0 | TimeTicks | sysUpTime of the reporting device |
| 1.3.6.1.6.3.1.1.4.1.0 | OID | snmpTrapOID — 1.3.6.1.4.1.99999.1.5 |
| 1.3.6.1.4.1.99999.2.9 | OctetString | cimRackId — rack identifier string |
| 1.3.6.1.4.1.99999.2.10 | Gauge32 | cimRackDownCount — number of impaired devices |

## 3.14  Humidity Alert

| OID | 1.3.6.1.4.1.99999.1.6 |
| --- | --- |
| Severity | MAJOR / CRITICAL |
| Applicable Devices | Environmental Sensor devices only (Raritan DPX2, Vertiv Geist, APC NetBotz) |

Relative humidity is outside the ASHRAE-recommended safe operating range of 30–70%. Three rules target this trap: SensorLowHumidity (< 30%, major), SensorHighHumidity (> 70%, major), and SensorCriticalHumidity (> 80%, critical). Humidity values are transmitted as integer percentages in the varbind.

SNMPv2c Varbinds:

| OID | Type | Description |
| --- | --- | --- |
| 1.3.6.1.2.1.1.3.0 | TimeTicks | sysUpTime |
| 1.3.6.1.6.3.1.1.4.1.0 | OID | snmpTrapOID — 1.3.6.1.4.1.99999.1.6 |
| 1.3.6.1.4.1.99999.2.12 | Gauge32 | cimHumidity — current relative humidity % (integer) |
| 1.3.6.1.4.1.99999.2.15 | Gauge32 | cimHumidityThreshold — threshold % (30 low / 70 high / 80 critical) |

## 3.15  Dew Point Alert

| OID | 1.3.6.1.4.1.99999.1.7 |
| --- | --- |
| Severity | CRITICAL |
| Applicable Devices | Environmental Sensor devices only — Vertiv Geist GTHD/IMD-3 sensors |

The dew-point temperature exceeded 21°C, indicating a condensation risk. Condensation on cold metal surfaces (heat exchangers, server chassis) can cause corrosion and short circuits. This is modeled after the Vertiv Geist GTHD sensor which reports dew-point directly. Values are encoded as integer tenths of °C (value × 10) for sub-degree precision without floating point.

SNMPv2c Varbinds:

| OID | Type | Description |
| --- | --- | --- |
| 1.3.6.1.2.1.1.3.0 | TimeTicks | sysUpTime |
| 1.3.6.1.6.3.1.1.4.1.0 | OID | snmpTrapOID — 1.3.6.1.4.1.99999.1.7 |
| 1.3.6.1.4.1.99999.2.13 | Gauge32 | cimDewpoint — dew-point °C × 10 (e.g. 215 = 21.5°C) |
| 1.3.6.1.4.1.99999.2.16 | Gauge32 | cimDewpointThreshold — 210 (21.0°C × 10) |

## 3.16  Airflow Alert

| OID | 1.3.6.1.4.1.99999.1.8 |
| --- | --- |
| Severity | MAJOR / CRITICAL |
| Applicable Devices | Environmental Sensor devices only — APC NetBotz 250/355 sensors |

Airflow velocity is outside the safe operating band of 0.3–3.5 m/s. Low airflow (< 0.3 m/s, critical) indicates a blocked air path, failed CRAC unit, or hot-aisle/cold-aisle containment breach. High airflow (> 3.5 m/s, major) may indicate a pressure imbalance or failed damper. Values are encoded as integer tenths of m/s (value × 10).

SNMPv2c Varbinds:

| OID | Type | Description |
| --- | --- | --- |
| 1.3.6.1.2.1.1.3.0 | TimeTicks | sysUpTime |
| 1.3.6.1.6.3.1.1.4.1.0 | OID | snmpTrapOID — 1.3.6.1.4.1.99999.1.8 |
| 1.3.6.1.4.1.99999.2.14 | Gauge32 | cimAirflow — airflow m/s × 10 (e.g. 28 = 2.8 m/s) |
| 1.3.6.1.4.1.99999.2.17 | Gauge32 | cimAirflowThreshold — 35 (3.5 m/s × 10) high / 3 (0.3 m/s × 10) low |

# 4. Shipped Rule Listing

The following rules are shipped in trap_rules.json and loaded at startup. All rules are enabled by default and can be toggled individually via the Rules Panel or by editing trap_rules.json. Priority values are evaluated highest-first; rules with equal priority fire in declaration order.

| Rule Name | Type | Condition | Trap OID | Severity | Pri | Device Types |
| --- | --- | --- | --- | --- | --- | --- |
| LinkDown | State Change | interface_status: up → down | 1.3.6.1.6.3.1.1.5.3 | MAJOR | 200 | All |
| LinkUp | State Change | interface_status: down → up | 1.3.6.1.6.3.1.1.5.4 | INFORMATIONAL | 200 | All |
| OOBSwitchLinkDown | State Change | interface_status: up → down | 1.3.6.1.6.3.1.1.5.3 | CRITICAL | 210 | oob_switch |
| OOBSwitchLinkUp | State Change | interface_status: down → up | 1.3.6.1.6.3.1.1.5.4 | INFORMATIONAL | 210 | oob_switch |
| LinkFlap | Temporal | linkDown ≥ 3× in 60 s window | 1.3.6.1.4.1.99999.1.4 | CRITICAL | 170 | All |
| HighCPU | Threshold | cpu_usage > 90% | 1.3.6.1.4.1.99999.1.1 | MAJOR | 150 | All |
| HighCPUSustained | Threshold | cpu_usage > 90% for 300 s | 1.3.6.1.4.1.99999.1.1 | CRITICAL | 160 | All |
| CPUNormal | Threshold | cpu_usage < 70% [recovery of HighCPU] | 1.3.6.1.6.3.1.1.5.4 | INFORMATIONAL | 100 | All |
| CriticalCPUAndTemp | Composite AND | cpu_usage > 90% AND temperature > 60°C | 1.3.6.1.4.1.99999.1.1 | CRITICAL | 200 | All |
| HighMemory | Threshold | memory_usage > 85% | 1.3.6.1.4.1.99999.1.2 | MAJOR | 150 | All |
| MemoryNormal | Threshold | memory_usage < 70% [recovery of HighMemory] | 1.3.6.1.6.3.1.1.5.4 | INFORMATIONAL | 100 | All |
| HighTemperature | Threshold | temperature > 60°C | 1.3.6.1.4.1.99999.1.3 | CRITICAL | 180 | All |
| TemperatureNormal | Threshold | temperature < 55°C [recovery of HighTemperature] | 1.3.6.1.6.3.1.1.5.4 | INFORMATIONAL | 100 | All |
| UPSOnBattery | State Change | ups_status: normal → on_battery | 1.3.6.1.2.1.33.2.0.1 | CRITICAL | 200 | ups |
| UPSLowBattery | State Change | ups_status: any → low_battery | 1.3.6.1.2.1.33.2.0.2 | CRITICAL | 200 | ups |
| BGPSessionDown | State Change | bgp_session: established → idle | 1.3.6.1.2.1.15.0.2 | CRITICAL | 190 | router, firewall |
| RackFailure | Rack Failure | ≥ 3 devices in same rack impaired | 1.3.6.1.4.1.99999.1.5 | CRITICAL | 190 | All |
| SensorAmbientTempHigh | Threshold | ambient_temp > 32°C | 1.3.6.1.4.1.99999.1.3 | MAJOR | 180 | sensor |
| SensorAmbientTempCritical | Threshold | ambient_temp > 38°C | 1.3.6.1.4.1.99999.1.3 | CRITICAL | 185 | sensor |
| SensorAmbientTempNormal | Threshold | ambient_temp < 28°C [recovery] | 1.3.6.1.6.3.1.1.5.4 | INFORMATIONAL | 100 | sensor |
| SensorHighHumidity | Threshold | humidity > 70% | 1.3.6.1.4.1.99999.1.6 | MAJOR | 175 | sensor |
| SensorCriticalHumidity | Threshold | humidity > 80% | 1.3.6.1.4.1.99999.1.6 | CRITICAL | 180 | sensor |
| SensorLowHumidity | Threshold | humidity < 30% | 1.3.6.1.4.1.99999.1.6 | MAJOR | 175 | sensor |
| SensorHumidityNormal | Composite AND | humidity ≥ 30% AND ≤ 70% [recovery] | 1.3.6.1.6.3.1.1.5.4 | INFORMATIONAL | 100 | sensor |
| SensorHighDewPoint | Threshold | dewpoint > 21°C | 1.3.6.1.4.1.99999.1.7 | CRITICAL | 185 | sensor |
| SensorDewPointNormal | Threshold | dewpoint ≤ 17°C [recovery] | 1.3.6.1.6.3.1.1.5.4 | INFORMATIONAL | 100 | sensor |
| SensorHighAirflow | Threshold | airflow > 3.5 m/s | 1.3.6.1.4.1.99999.1.8 | MAJOR | 170 | sensor |
| SensorLowAirflow | Threshold | airflow < 0.3 m/s | 1.3.6.1.4.1.99999.1.8 | CRITICAL | 180 | sensor |
| SensorAirflowNormal | Composite AND | airflow ≥ 0.3 AND ≤ 3.5 m/s [recovery] | 1.3.6.1.6.3.1.1.5.4 | INFORMATIONAL | 100 | sensor |

# 5. Separated Management Infrastructure

Prior versions modeled all device connectivity on a single network plane. v2.1 introduces a fully isolated out-of-band (OOB) management network as a first-class concept in both the topology data model and the visualization. This reflects real datacenter architectures where SNMP polling, gNMI telemetry, and console access traverse a physically separate management network that remains reachable even when the production network is impaired.

## 5.1  Dual IP Address Model

Every device now carries two optional IP addresses. The production ip_address (10.0.0.0/8 range) is used for data-plane traffic, ICMP reachability checks, and topology visualization. The management mgmt_ip (192.168.0.0/16 range) is the address over which SNMP (UDP 161), gNMI (gRPC), and trap delivery operate. Devices without a mgmt_ip set do not participate in the management layer.

## 5.2  New Management Device Types

| OOB Switch | Provides layer-2 aggregation for all management connections. Connects to every device in the topology via the management layer. Participates in the rule engine for link-down/link-up and CPU events. Models: Cisco Catalyst 1000-48T / 24T HPE Aruba 2530-48G / 2530-24G Dell N1148T-ON / N1124T-ON |
| --- | --- |

| UPS (Uninterruptible Power Supply) | Provides battery-backed power to rack equipment. Simulates mains failure events (on_battery) and battery exhaustion (low_battery). Capacities from 1.5 kVA (edge rack) to 100 kVA (datacenter floor). Fires UPS_ON_BATTERY and UPS_LOW_BATTERY traps using RFC 1628 OIDs. Models: APC Smart-UPS 1500 / 3000 / SRT 5000, Symmetra PX 100 Eaton 5PX 2200 / 9PX 5000 / 9E 20 kVA / 93E 40 kVA Vertiv Liebert GXT5 2000VA / EXL S1 20 kVA / APS 20 kVA |
| --- | --- |

| Rack PDU | Switched and metered power distribution within a rack (0U / 1U form factor). 12–48 C13/C19 outlets. Participates in the management layer for SNMP access and in the power layer as a dependency node. A PDU failure is the primary trigger for the Rack Failure rule. Models: APC AP8941 / AP8886 / AP8959 / AP8681 Raritan PX3-5190R / PX3-5161R / PX2-5170CR Eaton ePDU G3 MA / MI variants Vertiv Geist rPDU2 15A / 30A Server Technology Sentry PT40 / 4805-XLS |
| --- | --- |

| Floor PDU / RPP | 3-phase power distribution panel feeding multiple rack PDUs. Capacities 24–160 kVA. Sits at the top of the power hierarchy below the mains utility feed. A floor PDU failure can take down an entire row of racks. Models: APC FlexPDU 40 kVA / Galaxy RPP 80A Eaton PDU 80 kVA / 160 kVA Vertiv Liebert MPX 60 kVA / MPH2 24 kVA Raritan PX3-5000 Floor 30A |
| --- | --- |

| Environmental Sensor | Monitors ambient datacenter conditions. Reports temperature, humidity, dew-point, and airflow via dedicated OIDs modeled after real vendor MIBs. Drives the full suite of environmental trap rules. Models: Raritan DPX2-T1H1 / DPX2-T3H1 Vertiv Geist GTHD / IMD-3 APC NetBotz 250 / 355 |
| --- | --- |

# 6. Power Infrastructure

v2.1 introduces initial support for modeling the datacenter power distribution hierarchy. Power topology is modeled as a separate network layer with directed dependency edges. This enables cross-layer event correlation — a power device failure can be linked to the network impact it causes via the Rack Failure rule.

Power distribution hierarchy:

Utility Feed (Mains)

└── Floor PDU / RPP  (3-phase, 80–160 kVA)

└── Rack PDU   (12–48 outlets, C13/C19)

└── UPS  (battery backup, per-rack)

└── Network devices / Servers

Power layer edges in the topology JSON carry layer="power". The topology engine stores these edges in the NetworkX MultiGraph alongside production and management edges. The visualization renders power edges as dotted orange lines, distinguishing them visually from the data and management planes.

Each device node in the topology may carry a power_source field pointing to its upstream PDU node ID, and an ups_backup field pointing to its UPS node ID. These relationships allow the rule engine's Rack Failure evaluator to reason about shared power dependencies when counting impaired devices per rack.

## 6.1  Power Device Metrics

| Device Type | Metric | Description |
| --- | --- | --- |
| UPS | ups_status | normal ¦ on_battery ¦ low_battery |
| UPS | temperature | Chassis temperature °C |
| PDU | power_draw_w | Total outlet load in watts |
| Floor PDU | power_draw_w | Aggregate branch circuit load |
| Sensor | ambient_temp | Inlet air temperature °C |
| Sensor | humidity | Relative humidity % |
| Sensor | dewpoint | Dew-point °C (Vertiv Geist only) |
| Sensor | airflow | Airflow m/s (APC NetBotz only) |

# 7. Physical Location via SNMP

Devices carry structured physical location metadata that is surfaced both in topology JSON and through SNMP-polled data. This enables NMS platforms to correlate network events with physical datacenter locations.

| Field | Description |
| --- | --- |
| rack_id | Composite identifier: datacenter:row:rack, e.g. DC1:R1:RACK3. Used by the Rack Failure rule for device grouping. |
| datacenter | Datacenter name, e.g. DC1, DC2. Matches the topology's datacenter field. |
| rack_row | Numeric row identifier within the datacenter room. |
| rack_num | Rack number within the row. |
| rack_unit | Starting U position of the device within the rack (1 = bottom). |
| room | Room name within the datacenter, e.g. Core, Pod-A, MDF. |
| datacenter_city | City where the datacenter is located, e.g. Dallas, New York. |
| country | Country, e.g. USA, UK. |

## 7.1  LLDP-Based Location Discovery

The SNMPRec generator populates LLDP-MIB neighbor tables in each device's .snmprec dataset based on the configured topology adjacency. When an NMS tool walks LLDP on any device, it receives accurate neighbor records — including the neighbor's management IP — which can be cross-referenced with DCIM data to infer physical location.

| OID | MIB Object | Value |
| --- | --- | --- |
| 1.0.8802.1.1.2.1.4.1.1.9.{p}.{r} | lldpRemSysName | Neighbor device hostname |
| 1.0.8802.1.1.2.1.4.1.1.5.{p}.{r} | lldpRemChassisId | Neighbor chassis MAC address |
| 1.0.8802.1.1.2.1.4.1.1.7.{p}.{r} | lldpRemPortId | Neighbor port name |
| 1.0.8802.1.1.2.1.4.2.1.3.{p}.{r}.{s} | lldpRemManAddr | Neighbor management IP (mgmt_ip preferred, then ip_address) |
| 1.3.6.1.4.1.9.9.23.1.2.1.1.6.{p}.{r} | cdpCacheDeviceId | CDP neighbor hostname (Cisco devices) |
| 1.3.6.1.4.1.9.9.23.1.2.1.1.4.{p}.{r} | cdpCacheAddress | CDP neighbor IP |
| 1.3.6.1.4.1.9.9.23.1.2.1.1.7.{p}.{r} | cdpCacheDevicePort | CDP neighbor port name |

# 8. Three-Layer Topology Visualization

The topology canvas renders all three network planes simultaneously on the same interactive view. Each layer uses a distinct visual style so they can be distinguished at a glance, and each can be independently shown or hidden using the layer filter buttons in the toolbar overlay.

| Layer | Color | Line Style | Represents |
| --- | --- | --- | --- |
| Production | Gray (#7F8C8D) | Solid 2 px | Data-plane fabric links between network devices and servers |
| Management | Cyan (#00B4D8) | Dashed 1.5 px | OOB telemetry paths, SNMP and gNMI access routes |
| Power | Orange (#F59E0B) | Dotted 1.5 px | Power distribution dependency chains |

## 8.1  Device Node Color Scheme

| Device Type | Hex Color | Color Name | Icon Label |
| --- | --- | --- | --- |
| Router | #2980B9 | Blue | R |
| Switch | #27AE60 | Green | S |
| Server | #8E44AD | Purple | SRV |
| Firewall | #E67E22 | Orange | FW |
| Load Balancer | #16A085 | Teal | LB |
| UPS | #C9A227 | Gold | UPS |
| Rack PDU | #D44F00 | Dark Red | PDU |
| Floor PDU | #B03060 | Magenta | FPDU |
| OOB Switch | #7C3AED | Indigo | OOB |
| Environmental Sensor | #0891B2 | Cyan | SNS |

## 8.2  Canvas Features

Drag-and-drop node repositioning — topology layout persists to JSON

Right-click node context menu: view properties, edit device, delete

Right-click link context menu: view interface details, break link, restore link, remove

Broken link visualization: red X overlay and 10% opacity fade on affected edge

Layer filter toolbar: All / Prod / Mgmt / Power toggle buttons

Search bar: real-time name and IP matching with match count display

Zoom controls: ±1.15× step, mouse wheel, fit-to-view

Live SNMP Discovery button: triggers LLDP walk and displays results

Link tooltip on hover: shows interface speeds, neighbor names, layer type

# 9. Included Topology Templates

Six pre-built topology files are included. All have been updated to include management and power layers, UPS and PDU nodes, OOB switches, and environmental sensors. Each topology file sets the metadata flags has_management_layer: true and has_power_layer: true.

| File | Description |
| --- | --- |
| large_datacenter_3tier.json | Classic three-tier architecture: core, distribution, and access layers with full redundancy |
| large_datacenter_spine_leaf.json | Modern Clos fabric with spine and leaf tiers, ideal for east-west traffic workloads |
| large_hyperscale_pod.json | Hyperscale pod architecture modeled after large-scale cloud datacenter deployments |
| large_enterprise_wan.json | WAN-centric topology with edge routers, firewalls, and branch interconnects |
| large_4dc_enterprise.json | Four-datacenter enterprise topology with inter-DC WAN links and shared services |
| dual_dc_enterprise.json | Active/active dual-datacenter setup with full management and power layer coverage |

# 10. Configuration Reference

## 10.1  trap_rules.json

User-defined rules are loaded from trap_rules.json at startup and merged with the built-in defaults. The file is a JSON array of rule objects. Changes take effect on the next application start or when reloaded via the Rules Panel.

{

"rule_name":   "HighCPUSustained",

"trap_oid":    "1.3.6.1.4.1.99999.1.1",

"severity":    "critical",

"enabled":     true,

"priority":    160,

"device_types": [],

"condition": {

"condition_type": "threshold",

"metric":         "cpu_usage",

"operator":       ">",

"threshold":      90.0,

"duration_sec":   300.0

}

}

| Field | Type | Description |
| --- | --- | --- |
| rule_name | string | Unique rule identifier; used by recovery_of references |
| trap_oid | string | OID of the trap to fire when the rule matches |
| severity | string | informational ¦ minor ¦ major ¦ critical |
| enabled | boolean | Whether the rule is active (can be toggled at runtime) |
| priority | integer | Evaluation order — higher value fires first |
| device_types | array | Restrict rule to listed device types; empty array = all types |
| is_recovery | boolean | Marks this rule as a clearance rule |
| recovery_of | string | rule_name of the alert this rule clears |
| condition | object | Condition object — see Section 2.2 for type-specific fields |

## 10.2  Topology JSON — New Fields in v2.1

| Field | Location | Description |
| --- | --- | --- |
| mgmt_ip | Device node | Management-plane IPv4 address for SNMP/gNMI polling |
| rack_id | Device node | Physical rack identifier, e.g. DC1:R1:RACK3 |
| rack_row / rack_num | Device node | Numeric row and rack position within the datacenter |
| power_source | Device node | Node ID of the upstream PDU providing power |
| ups_backup | Device node | Node ID of the UPS providing battery backup |
| layer | Edge | production ¦ management ¦ power |
| metadata.has_management_layer | Top-level | Boolean — enables management layer rendering and discovery |
| metadata.has_power_layer | Top-level | Boolean — enables power layer rendering |

## 10.3  Trap Receiver

The SNMP trap receiver address and port are configured in the Trap Panel. The default is 127.0.0.1:162. Any SNMPv2c trap receiver (e.g., Zabbix, Nagios, Prometheus SNMP Exporter, Net-SNMP snmptrapd) can receive traps from the simulator without modification.

## 10.4  Upgrade Notes

Topologies created with v2.0 are fully compatible with v2.1. Devices without a mgmt_ip field will not participate in the management layer. Devices without a rack_id field will not be grouped by the Rack Failure rule. Custom trap_rules.json files from v2.0 are compatible without modification. No data migration is required.